SANSKRITI LABS

Snaprint — Self-Service Print Kiosk

Procurement & Product Specification

Snaprint S1 — Kiosk Enclosure

Date: 30 July 2026

Prepared by: Goutham, Sanskriti Labs

Confidential — for use between Sanskriti Labs and Ratnam Solutions Private Limited only. This document and its contents (dimensions, component specifications, and internal design details) are shared solely to enable an accurate manufacturing quote for the Snaprint S1, and should not be disclosed, copied, or shared with any third party without Sanskriti Labs' prior written consent.

## 1. Project overview

Sanskriti Labs is developing the Snaprint S1, a self-service print kiosk for deployment in partner print shops near college campuses. This document sets out the confirmed dimensions, internal layout, and component list for the kiosk housing, verified against the actual printer and touchscreen hardware, so an accurate manufacturing quote can be prepared.

## 2. Kiosk housing specification

| Item | Footprint (W x D) | Height |
| --- | --- | --- |
| Kiosk housing | 55 x 70 cm | 176.8 cm |

The 70 cm depth gives adequate clearance behind the printer for the rear service door and cable routing — see Section 4 for full component dimensions.

## 3. Internal layout (floor to top)

| Zone | Height | Contents |
| --- | --- | --- |
| Base | 0–90 cm | Structural base; locking castors or fixed feet; recommended location for a weighted base plate (see stability note below) |
| Output & routing | 90–112 cm | 7 cm print collection slot, plus a 15 cm internal gap routing the paper path down from the printer |
| Screen + printer | 112–162 cm | 15.6" touchscreen mounted on the front face; Epson WF-C5890 printer mounted directly behind it |
| Upper body | 162–176.8 cm | Branding graphic; full-height LED edge stripe continues to the top |

Stability note: positioning the printer at screen height rather than low in the cabinet raises the unit's centre of gravity. A weighted base plate in the base zone is recommended to keep the unit stable in an unattended public setting.

## 4. Component compatibility

| Component | Spec / model | Dimensions (W x D x H) | Supply |
| --- | --- | --- | --- |
| Printer | Epson WorkForce Pro WF-C5890 | 42.4 x 57.9 x 45.0 cm — operating configuration | Supplied by Sanskriti Labs |
| Touchscreen | 15.6" industrial Android touch display (exact model TBC) | Active area approx. 34.4 x 19.4 cm; estimated unit approx. 38 x 23 x 4 cm | Supplied by Sanskriti Labs |
| Exhaust fan | 12V DC axial fan with digital thermostat module (e.g. 3" fan + XH-W1209 controller, or equivalent) — reference example, confirm final sourcing | Fan approx. 76 x 76 x 25–30 mm | To confirm with Ratnam |

Printer dimensions are given in its operating configuration (output tray extended), since the kiosk runs unattended and the printer must stay ready to print at all times — not its storage/closed configuration, which is smaller. Touchscreen dimensions are estimated from the standard 15.6" 16:9 panel size; please confirm the exact model before finalising the front bezel cut-out.

## 5. Access, ventilation & cabling

- Front: no service door.

- Rear: one lockable service door sized for printer paper and ink refill.

- Side: one cutout for power and data cable entry, with the surge protection unit mounted at the entry point.

- Ventilation: a modest requirement. The Epson WF-C5890 draws approximately 27W standalone (PrecisionCore Heat-Free inkjet, not laser), so ventilation is mainly to manage moisture rather than a significant heat load. Recommend one small 12V DC exhaust fan (approx. 76 mm / 3") paired with a digital thermostat module (e.g. XH-W1209 or equivalent) with an NTC sensor, so the fan switches on automatically only once the compartment warms past a set point and off again once it cools — it should not run continuously. Placement should avoid the rear door's hinge and swing path.

## 6. Material & finish

- Enclosure: metal, powder-coated.

- Finish specifics (gauge, exact colour match to Signal Red #E63946 / Charcoal #111110 / Off-white #F5F5F5) at the manufacturer's standard build and recommendation.

## 7. Scope for vendor quote

- Kiosk housing fabrication to 55 x 70 cm footprint, 176.8 cm height

- Internal mounting structure for the Epson WF-C5890 printer (operating configuration)

- Front bezel cut-out and mount for the 15.6" touchscreen

- Rear lockable service door sized for printer paper/ink access

- Side cable entry cutout with a surge protector mounting point

- Cable management and routing between printer, screen, and power unit

- Ventilation for the printer compartment, including a mounting point for a small thermostatically-controlled exhaust fan (approx. 76 mm / 3")

- Power distribution — single mains input, internally distributed

- Base with locking castors or fixed feet, plus a weighted base plate for stability

- Powder-coated metal finish, brand colour match (Signal Red #E63946, Charcoal #111110, Off-white #F5F5F5)

## 8. Requirements for an accurate quote

- Confirm the exact touchscreen make/model once selected, for a precise bezel cut-out

- Confirm whether 70 cm depth gives adequate rear-door and cable clearance, or advise if more is needed

- Confirm sourcing and exact mounting position for the exhaust fan and thermostat module

- Advise whether locking castors or fixed levelling feet are your standard build for a unit this height

- Unit quantity (single unit / pilot batch)

- Lead time from confirmation to delivery

- Delivery and installation charges, if separate from unit cost

- Warranty terms on the housing and fabrication

- Any design changes needed given the confirmed dimensions above

Happy to hop on a call if any of the above needs clarification before you finalise the quote.